| Title: | Innovative Design and Scalable Manufacturing of a High-Performance Globe Valve with Plug-Type Disc |  |  |
| --- | --- | --- | --- |
| Author(s) Name: | Mohammad Musfeq Iqfath, Sakib Mahmud, *Md Mahadi Hasan |  |  |
| Contact Email(s): | mahadi@aiub.edu |  |  |
| Published Journal Name: | Journal of Engineering and Sciences |  |  |
| Type of Publication: | Journal |  |  |
| Volume: | 55 | Issue | 1 |
| Publisher: | Dhaka University |  |  |
| Publication Date: | Sep 29, 2025 |  |  |
| ISSN: | 2356-8550 |  |  |
| DOI: | 10.21608/jesaun.2025.399108.1585 |  |  |
| URL: | https://jesaun.journals.ekb.eg/article_455653.html |  |  |
| Other Related Info.: | Page |  |  |

| Abstract: |  |
| --- | --- |
| This study presents the design and development of an advanced industrial-grade globe valve for enhanced flow control, excellent sealing performance, and augmented longevity. A key scientific contribution of this research is the implementation of an innovative plug-type disc design that demonstrates a 27% reduction in leakage compared to conventional flat-disc configurations, as validated through computational fluid dynamics (CFD) simulations. The Z-type valve design optimizes seating alignment, reduces pressure drop by 20 %, and extends operational lifespan through strategic material selection (ASTM A48 cast iron body, AISI 304 stainless steel internals) and a three-layered sealing mechanism. The manufacturing process integrates precision sand casting, CNC machining, and rigorous testing (ISO 5208:2015), yielding a 95% casting success rate and a 23.82% cost reduction (26 USD/unit) compared to imported valves (47 USD–63 USD). Validated in Bangladesh’s industrial context, the valve demonstrates superior throttling, bubble-tight shutoff, and flow stability, reducing import dependency and supporting sustainable manufacturing. This scalable framework positions the valve for global applications in power generation, chemical processing, and oil/gas industries. |  |