| Title: | Automated Image Processing and Machine-Learning Based System for Efficient Dust Detection and Cleaning of Solar Panels |  |  |  |
| --- | --- | --- | --- | --- |
| Author(s) Name: | Rethwan Faiz, Md Araf Ohid, Nuzat Nuary Alam, Mohammad Hasan Imam |  |  |  |
| Contact Email(s): | rethwan_faiz@aiub.edu |  |  |  |
| Published Journal Name: | IEEJ Journal of Industry Applications |  |  |  |
| Type of Publication: | Journal |  |  |  |
| Volume: | 15 |  | Issue | 3 |
| Publisher: | The Institute of Electrical Engineers of Japan |  |  |  |
| Publication Date: | 1st May 2026 |  |  |  |
| ISSN: | Online ISSN : 2187-1108 Print ISSN : 2187-1094 ISSN-L : 2187-1094 |  |  |  |
| DOI: | https://doi.org/10.1541/ieejjia.25003095 |  |  |  |
| URL: | https://www.jstage.jst.go.jp/article/ieejjia/15/3/15_25003095/_article/-char/ja/ |  |  |  |
| Other Related Info.: |  |  |  |  |
| Abstract: |  |  |  |  |
| Solar photovoltaic (PV) technology is a promising renewable energy source; however, its efficiency considerably reduces dust accumulation, particularly in arid or polluted environments. Traditional water-based or manual cleaning methods are inefficient, costly, and unsustainable. This study proposes an automated dry-cleaning system that integrates machine learning and image processing for real-time dust detection and targeted cleaning. The system employs Canny edge detection and YOLOv5 to isolate PV panel regions and classify dust severity, labeling panels with ≥30% surface coverage as “dusty.” Arduino Mega and Jetson Nano jointly control a brush-based mechanism driven by dual DC gear motors, optimized for vertical (400rpm) and horizontal (200rpm) cleaning. Experimental evaluation on 15-W PV modules demonstrated that the system improved module efficiency from 77.1% under dusty conditions to 96.0% after cleaning, corresponding to an average 39% increase in output power. The dust detection algorithm achieved 82% classification accuracy, and the optimal cleaning performance was obtained with two sweeps (≈1min 59s), beyond which additional passes yielded negligible gains. By integrating accurate detection, optimized automation, and sustainable dry-cleaning, the proposed framework provides a scalable theresource-efficient solution for maintaining long-term PV performance. |  |  |  |  |