| Title: | A Single-Band 28.5GHz Rectangular Microstrip Patch Antenna for 5G Communications Technology |  |  |
| --- | --- | --- | --- |
| Author(s) Name: | Fardeen Mahbub; Rashedul Islam; Sayed Abdul Kadir Al-Nahiun; Shouherdho Banerjee Akash; Raja Rashidul Hasan; Md. Abdur Rahman |  |  |
| Contact Email(s): | hemal@aiub.edu |  |  |
| Published Conference Name: | 2021 IEEE 11th Annual Computing and Communication Workshop and Conference (CCWC) |  |  |
| Type of Publication: | Conference |  |  |
| Volume: | NA | Issue | NA |
| Publisher: | NA |  |  |
| Publication Date: | March 2021 |  |  |
| ISSN: | NA |  |  |
| DOI: | 10.1109/CCWC51732.2021.9376047 |  |  |
| URL: | https://ieeexplore.ieee.org/document/9376047 |  |  |
| Other Related Info.: | NA |  |  |

| Abstract: |  |
| --- | --- |
| The popularity, demand, and usefulness of 5G Communication Systems are rapidly increasing. Since the future of 5G Communication systems requires higher gain and efficiency, a rectangular microstrip patch antenna has been designed in this paper. The proposed model has a better return loss and a healthy efficiency characteristic. The operating frequency of 28.5 GHz (Ka-band) was used here, one of the prominent frequency bands for 5G communication. The Patch's architectural structure is 7.885 mm*8.935 mm*0.5 mm, with the help of a return loss of -48.309 dB, a gain of 7.425 dB, VSWR of 1.007129, and directivity of 8.141 dBi have been achieved in the proposed model. Also, the efficiency of 91.16%, Bandwidth of 1.2 GHz, and a surface current of 760.4 A/m were successfully achieved, which is also helpful for faster 5G communication. Considering all these mentioned parameters, the designed antenna can be suitable for 5G communication technology soon. |  |