| Title: | Micromanufacturing Technology and its Practice |  |  |
| --- | --- | --- | --- |
| Author(s) Name: | Zhengyi Jiang, Mahadi Hasan, Hamidreza Kamalic, Fanghui Jia, Haibo Xie |  |  |
| Contact Email(s): | mahadi@aiub.edu |  |  |
| Published Journal Name: | Materials and Technologies in Precision Machinery III, Trans Tech Publications Ltd |  |  |
| Type of Publication: | Book Chapter |  |  |
| Volume: | 311 | Issue | 1 |
| Publisher: | Transtec Publications |  |  |
| Publication Date: | Oct 5, 2022 |  |  |
| ISSN: | 1662-9779 |  |  |
| DOI: | 10.4028/www.scientific.net/SSP.311.12 |  |  |
| URL: | https://www.scientific.net/SSP.311.12 |  |  |
| Other Related Info.: | Page 12-20 |  |  |

| Abstract: |  |
| --- | --- |
| In order to make micro composite drills (Fig. 1), cemented tungsten carbide (WC-10Co) and high strength (AISI 4340) steel were successfully bonded by hot compaction diffusion bonding at a low temperature. The effects of holding time, pressure and temperature on microstructure and mechanical properties of the sintered carbides and bonding strengths of the bimetallic composites were examined, and a transitional layer was found at the interface as a result of elemental inter-diffusion. The optimal bonding parameters were determined to achieve the maximum bonding strength of 226 MPa of the WC-10Co/AISI 4340 steel joints, which is helpful in producing micro composite drills. Microforming is introduced to produce lighter and more energy effective products. In this study, Magnesium-Lithium (Mg-Li) alloy, new material in microscale, was chosen to superior formed micro-cup due to its ultralight weight with outstanding ductility. The dry and oil lubrication conditions were chosen as benchmarks to investigate effects of a novel oil-based nanoparticle lubricant in micro deep drawing (MDD) process of Mg-Li alloy. Finite Element (FE) modelling was conducted and the simulation results of the drawing force were in a good agreement with the experimental results. The formed cup quality with consideration on the surface roughness has been extensively evaluated and the results illustrated the quality improvement was substantial. |  |