| Title: | Thermal Confinement by Monolayer MoS2 for Reduced RESET Current in Phase Change Memory Pillar Cells |  |  |
| --- | --- | --- | --- |
| Author(s) Name: | Sadid Muneer, Muhammad Aminul Haque Chowdhury, Md Kabiruzzaman, Shafat Shahnewaz, Nafisa Noor, Mainul Hossain |  |  |
| Contact Email(s): | kabiruzzaman@aiub.edu |  |  |
| Published Journal Name: | ACS Applied Electronic Materials |  |  |
| Type of Publication: | Journal |  |  |
| Volume: | 6 | Issue | 7 |
| Publisher: | American Chemical Society (ACS) |  |  |
| Publication Date: | June 18, 2024 |  |  |
| ISSN: | 2637-6113 |  |  |
| DOI: | https://doi.org/10.1021/acsaelm.4c00721 |  |  |
| URL: | https://pubs.acs.org/doi/abs/10.1021/acsaelm.4c00721 |  |  |
| Other Related Info.: | Page 5222–5229 |  |  |

| Abstract: |  |
| --- | --- |
| Phase change memory (PCM) is one of the most promising nonvolatile memory technologies for high-density, high-endurance, fast-switching, and multilevel data storage. However, the high RESET current requirement remains a critical bottleneck in the development of PCM technology. In this work, we propose a pillar-shaped PCM device that consists of a Ge2Sb2Te5 (GST) layer sandwiched between the top and the bottom TiN electrodes. An atomically thin layer of MoS2 is grown on top of the oxidized bottom TiN layer. A filament formed through the TiO2 and MoS2 layers enables electrical conduction, while the high thermal resistivity of MoS2 ensures excellent thermal confinement within the GST layer. Finite element simulations show a 91% reduction in RESET current brought about by the filament, while the use of MoS2 yields a further ∼30% decrease in the switching power. The results presented here demonstrate the potential use of two-dimensional (2D) materials with conventional PCM cells to reduce switching power. |  |