| Title: | Wavelength-Resolved Optical Attenuation in ZnO-Based Transparent Oxide Thin Films and Nanostructures for Optoelectronic Interfaces |  |  |
| --- | --- | --- | --- |
| Author(s) Name: | M. Reefaz Rahman, Mohammad Nasir Uddin, Tasnim Ashrafy, Md. Kabiruzzaman, Mahbubul Hoq |  |  |
| Contact Email(s): | kabiruzzaman@aiub.edu |  |  |
| Published Journal Name: | Journal of Electronic Materials |  |  |
| Type of Publication: | Journal |  |  |
| Volume: | N/A | Issue | N/A |
| Publisher: | American International University-Bangladesh (AIUB) |  |  |
| Publication Date: | September, 2026 |  |  |
| ISSN: | 1543-186X |  |  |
| DOI: | https://doi.org/10.1007/s11664-026-13155-8 |  |  |
| URL: | https://link.springer.com/article/10.1007/s11664-026-13155-8#citeas |  |  |
| Other Related Info.: | N/A |  |  |

| Abstract: |  |
| --- | --- |
| Transparent oxide thin films and nanostructures are often evaluated using average transmittance, optical bandgap, Urbach energy, electrical conductivity, and transparent-conductor figure of merit. However, optoelectronic interfaces require wavelength-specific optical access rather than broadband transparency alone. Here, discrete wavelength–transmittance coordinates from prior ZnO-based datasets are converted into a dopant- and morphology-resolved attenuation atlas. Undoped ZnO nanorod layers are compared with Mg-doped ZnO (MZO), Al-doped ZnO (AZO), and Sn-doped ZnO films. Transmittance is converted into total optical attenuation, or transmission-derived insertion loss, at 405 nm, 450 nm, 532 nm, and 637 nm and across the 650–800-nm red/near-infrared (NIR) band. This attenuation includes absorption, reflection, scattering, substrate effects, and interference rather than absorption alone. MZO 8 at.% gives the lowest extracted attenuation at 405 nm, AZO 2 at.% gives the lowest at 532 nm, and AZO 4 at.% gives the lowest at 637 nm and across 650–800 nm. The results are positioned against contemporary dopant-free and doped ZnO technologies produced by sputtering, chemical vapor deposition, pulsed laser deposition, spray pyrolysis, spatial atomic layer deposition, and solution processing. The framework converts conventional spectra into device-level attenuation metrics while retaining the thickness, morphology, and processing context required for realistic optoelectronic material selection.. |  |